EE277 Embedded System Design Course

LAB 3: AXI-Lite and GPIOs

- Contents

1	Introduction	1

1.1	Learning Outcomes	1

2	Details	1

2.1	Hardware	1

2.2	Memory Map	1

3	Procedure	1

3.1	Creating custom AXI4 Peripheral	5

3.2	Connecting AXI4 Peripheral to ZYNQ Processing System	11

3.3	Bitstream generation and Vitis coding	15

3.4	Troubleshooting	19

3.4.1	Makefile errors in Vitis	19

4	Tasks for this lab	21

# Introduction

## Learning Outcomes

In this lab, we will design and implement AXI-Lite peripheral to control General Purpose Input and Output Ports (GPIOs).

At the end of this module, you will be able to:

- Configure and integrate an AXI-Lite peripheral with a Cortex-A9 Processing System.

- Modify a C code to program the Cortex-A9 processor so that it reads the state of switches and control the LEDs.

- Demonstrate a functional simple system that lights up the LEDs based on the status of the switches.

# Details

## Hardware

DDR Controller

GPIO
Peripheral

LEDs

Switches

Cortex-A9

Interconnect

Programmable Logic

Processing System

Figure 1. Hardware system diagram.

## Memory Map

GPIO PERIPHERAL REGISTERS

| Register | Base address | Size |
| --- | --- | --- |
| Data | 0x43C0_0000 | 4 bytes |
| Direction | 0x43C0_0004 | 4 bytes |

# Procedure

Create a new workspace such that the path to the workspace does not contain any spaces. For Windows this could be a folder in C directory (C:/CreateADedicatedDirectory).

Open Xilinx Vivado and create a new project in your workspace as shown below:

Figure 2. Creating a project

Give it a meaningful name and click next. Select RTL Project and click “Next”. We will not be adding any sources or constraints so click “Next” for the next two pages. In the next page, click the “Boards” tab and search for “Zybo” in the search bar and click the download button next to “Zybo Z7-10” if the option is available. Then select it and click “Next” (Don’t click on the hyperlink but click on the empty area next to “Zybo Z7-10”).

Figure 3. Board Selection

Click “Finish”.

In the “Flow Navigator” menu on the left, click “Create Block Design” under “IP Integrator”:

Figure 4. Creating a new block design

Choose a design name or use the default one and click “Ok”.

Within the empty “Diagram” box on the right-hand side, right-click and select “Add IP”. Enter “Zynq” in the search box and choose “ZYNQ7 Processing System”. Click the “Run Block Automation” option that has now appeared and click “Ok” on the pop-up window.

Figure 5. Running Block Automation

Figure 6: Run Block Automation default settings

## Creating custom AXI4 Peripheral

Xilinx Vivado provides a simplified way to create an AXI4 peripheral using a wizard. Click “Tools” -> “Create and Package New IP”. Click “Next” and choose the following option:

Figure 9. Creating AXI4 peripheral

Give the following name to the IP (you may keep the IP location path as provided by default):

Figure 10. Adding peripheral details

We need a Subordinate interface and four 32-bit registers are enough for our switches and LEDs.

Figure 11. Peripheral settings for Subordinate interface

Finally, on the last page, choose “Edit IP” and click “Finish”. In the “Sources” window, you will see two Verilog files in Design Sources:

- A top file, i.e., AUP_advanced_SoC_v1_0.v

- A file that ends with “S00_AXI”, i.e., AUP_advanced_SoC_v1_0_S00_AXI.v

They are a basic template for an AXI-Lite peripheral and we can implement our GPIO logic by some modifications. Double-click to open the top-level Verilog file called “AUP_advanced_SoC_v1_0.v”:

Figure 12. Editing top-level Verilog file

Add the following lines of code below the comment “// Users to add ports here”:

// Users to add ports here

input wire [3:0] sw,

output wire [3:0] led,

Also add sw and led (underlined in red below) when instantiating the AXI Bus Interface S00_AXI, so that the code looks like the following:

// Instantiation of Axi Bus Interface S00_AXI

AUP_advanced_SoC_v1_0_S00_AXI # (

.C_S_AXI_DATA_WIDTH(C_S00_AXI_DATA_WIDTH),

.C_S_AXI_ADDR_WIDTH(C_S00_AXI_ADDR_WIDTH)

) AUP_advanced_SoC_v1_0_S00_AXI_inst (

.sw(sw),

.led(led),

.S_AXI_ACLK(s00_axi_aclk),

…

…

Save the changes in the file (Ctrl+S).

Next, expand and open the other Verilog file (AUP_advanced_SoC_v1_0_S00_AXI.v) shown below:

Figure 13: Edit the second Verilog file

Add the following lines of code below the comment “// Users to add ports here”:

// Users to add ports here

input wire [3:0] sw,

output wire [3:0] led,

Scroll down the file and search (Ctrl+F) for “// Address decoding for reading registers”. Then update the code so that it looks like the following (shown in red):

// Address decoding for reading registers

case ( axi_araddr[ADDR_LSB+OPT_MEM_ADDR_BITS:ADDR_LSB] )

2'h0   : reg_data_out <= {slv_reg0[C_S_AXI_DATA_WIDTH-1:4], sw[3:0]};

Scroll down the file and search (Ctrl+F) for “// // Add user logic here”. Then update the code so that it looks like the following (shown in red):

// Add user logic here

assign led[3:0] = slv_reg1[3:0];

Save the changes in the file (Ctrl+S).

Next, go to the “Package IP – AUP_advanced_SoC” tab, choose the “Customization Parameters” option on the left and click “Merge Changes  from Customization Parameters Wizard” to update the IP package with the changes we made in HDL files:

Figure 14. Saving all the changes

Click “Review and Package” on the left and select “Re-Package IP” at the bottom of the page. A prompt will appear – select “Yes” to close the project.

Go to the AXI4 peripheral folder you created earlier <IP location folder>\drivers\<AUP_advanced_SoC_v1_0>\src and locate a Makefile. In that Makefile, update the contents so that it is the following:

COMPILER=

ARCHIVER=

CP=cp

COMPILER_FLAGS=

EXTRA_COMPILER_FLAGS=

LIB=libxil.a

RELEASEDIR=../../../lib

INCLUDEDIR=../../../include

INCLUDES=-I./. -I${INCLUDEDIR}

INCLUDEFILES=*.h

LIBSOURCES= $(wildcard *.c)

OUTS = *.o

OBJECTS = $(addsuffix .o, $(basename $(wildcard *.c)))

ASSEMBLY_OBJECTS = $(addsuffix .o, $(basename $(wildcard *.S)))

libs:

echo "Compiling axi_gpio_asoc..."

$(COMPILER) $(COMPILER_FLAGS) $(EXTRA_COMPILER_FLAGS) $(INCLUDES) $(LIBSOURCES)

$(ARCHIVER) -r ${RELEASEDIR}/${LIB} ${OBJECTS} ${ASSEMBLY_OBJECTS}

make clean

include:

${CP} $(INCLUDEFILES) $(INCLUDEDIR)

clean:

rm -rf ${OBJECTS} ${ASSEMBLY_OBJECTS}

Then, click on “Settings” (under “Project Manager”) in the “Flow Navigator” menu on the left. Expand the “IP” section in the new window that appears and choose the “Repository” option.

Figure 15. Adding IP Repository

Click the “+” option under “IP Repositories” on the right and choose the AXI4 peripheral folder you created earlier (if it hasn’t already appeared) and click “Ok”. Right-click in the empty space of the “Diagram” box again and choose “Add IP”. Type “custom” in the search box and choose “custom_axi_gpio_asoc_v1_0” from the options. Click “Run Connection Automation” and then click “OK” to connect the AXI-Lite Subordinate interface on GPIO peripheral to the AXI Manager interface on ARM processor.

Figure 16: Connect AXI-Lite Subordinate interface (Custom IP) to AXI Manager interface

## Connecting AXI4 Peripheral to ZYNQ Processing System

Right-click on the empty space again in the diagram and choose “Create Ports” (Ctrl+K). Create 2 ports with the following settings and names:

Figure 17. Creating output port

Figure 18. Creating input port

Wire the “sw[3:0]” input to the “sw[3:0]” port of the “axi_gpio_asoc_0” block and the same for the “led[3:0]” output to the equivalent port of the block as shown in the diagram below. This connects them to external ports of the ZYNQ chip:

Figure 19. Vivado block diagram

Two IP blocks will be generated automatically. The “Processor System Reset” IP is used to generate reset signals for different peripherals. The “AXI Interconnect” IP here is used to interconnect AXI4-Lite Subordinate and AXI Manager.

Select the “Address Editor” tab next to “Diagram” and change the “64K” to “4K”. Save all your progress.

Figure 20. Changing peripheral address settings

Create a new file called “pins.tcl” using a text editor of your choice within your workspace. Add the following two lines of code within it. These tell Vivado to ignore the unspecified I/O Pin warnings. (article_56354)

set_property  SEVERITY {Warning} [get_drc_checks NSTD-1]

set_property  SEVERITY {Warning} [get_drc_checks UCIO-1]

Right-click the “Generate Bitstream” option in the “Flow Navigator” on the left and select “Bitstream settings”. Click the three dots next to “tcl.pre”:

Figure 21. Changing bitstream settings

Select the “New Script” option, click the three dots next to the empty box, choose the “pins.tcl” file you created earlier and click “Ok” on all windows.

Right-click the “Constraints” under the sources tab and select “Add sources”:

Figure 22. Adding sources

Select “Add or create constraints” and click “Next”. Select “Create File”, give any name to the file for example pin_constraints, and click “Finish”:

Figure 23. Creating a constraints file

Expand the “Constraints” folder within the “Sources” tab and double-click the file you just created to open it. Add the following constraints from “https://github.com/Digilent/Zybo-Z7-10-Pmod-VGA/blob/master/src/constraints/Zybo-Z7-Master.xdc”, and save the file:

Figure 24. Editing constraints file

Ensure that your XDC files have the following constraints uncommented:

##Switches

set_property -dict { PACKAGE_PIN G15   IOSTANDARD LVCMOS33 } [get_ports { sw[0] }]; #IO_L19N_T3_VREF_35 Sch=sw[0]

set_property -dict { PACKAGE_PIN P15   IOSTANDARD LVCMOS33 } [get_ports { sw[1] }]; #IO_L24P_T3_34 Sch=sw[1]

set_property -dict { PACKAGE_PIN W13   IOSTANDARD LVCMOS33 } [get_ports { sw[2] }]; #IO_L4N_T0_34 Sch=sw[2]

set_property -dict { PACKAGE_PIN T16   IOSTANDARD LVCMOS33 } [get_ports { sw[3] }]; #IO_L9P_T1_DQS_34 Sch=sw[3]

##LEDs

set_property -dict { PACKAGE_PIN M14   IOSTANDARD LVCMOS33 } [get_ports { led[0] }]; #IO_L23P_T3_35 Sch=led[0]

set_property -dict { PACKAGE_PIN M15   IOSTANDARD LVCMOS33 } [get_ports { led[1] }]; #IO_L23N_T3_35 Sch=led[1]

set_property -dict { PACKAGE_PIN G14   IOSTANDARD LVCMOS33 } [get_ports { led[2] }]; #IO_0_35 Sch=led[2]

set_property -dict { PACKAGE_PIN D18   IOSTANDARD LVCMOS33 } [get_ports { led[3] }]; #IO_L3N_T0_DQS_AD1N_35 Sch=led[3]

## Bitstream generation and Vitis coding

Under the “Sources” tab on the left, expand the “Design Sources” folder, right-click the design1.bd file, choose the “Create HDL Wrapper” and select all default options.

Figure 25. Creating HDL Wrapper

Save any other changes and click “Generate Bitstream” on the left and click “Ok” for all pop-ups. This process takes some time. Once the process is done, select “Open Hardware Manager”:

Figure 26. Generating bitstream and uploading to board

Connect the board and make sure the jumper (JP3) above the red LED on the Zybo board is in JTAG mode.

Then, in Vivado, click “Auto Connect” in the Hardware Manager as shown below:

Figure 27. Connecting the board

Right-click on the board, select “Program Device” as shown below and click “Program” on the pop-up window.

Figure 28. Programming the board

Once the board is programmed, the green LED labeled “LD12” should light up on the board. Click “File” on the main menu bar and select “Export” -> “Export Hardware” and click “Next” on the pop-up window. Choose the following option on the next page:

Figure 29. Exporting hardware and bitstream file

Choose the “export to” location as the project folder and save the file. Then click “Finish”.

Next, click “Tools” on the main menu bar and select “Launch Vitis IDE”. Choose the same project folder as your workspace. Click “File” -> “New” -> “Application Project”.

Figure 30. Creating a new application project

Select the “Create a new platform from hardware (XSA)” tab and click browse to select the XSA file you saved earlier:

Figure 31. Adding the XSA file

Click next and give a name (e.g. led_system) to the application project. Click “Next” until you reach the following page and choose “Empty Application(C)” and click “Finish”:

Figure 32. Creating an empty C Application

Then right-click the “src” folder within the application project you created and add a new file called “main.c”.

Figure 33. Adding a main.c file

Add the following code to the file and save all your changes:

volatile unsigned int value;

int main(){

while(1){

value = *(unsigned int*)0x43c00000;

*(unsigned int*)0x43c00004 = value;

}

}

From the address tab in Vivado, we know the switches are located at the address “0x43c00000”. All switches together are read as 4 bits with sw[3] being the MSB (most significant bit) and sw[0] as the LSB (least significant bit). If a switch is turned on then it’s corresponding bit will be 1, else it will be 0. We store the positions of the switches into a variable called “value” and write that into the address of the LEDs “0x43c00004” so that the LEDs mirror the On/Off state of the switches.

Right-click the application project in the explorer tab, select “Build Project” and ensure that the build is successful. Then right click again and select “Run As” and then “1 Launch Hardware” to upload everything to the board.

Figure 34. Running the program on the board.

Once this is done, you should be able to toggle the LEDs using the switches.

## Troubleshooting

### Makefile errors in Vitis

Double check if the 3 Makefiles in the project wrapper files which you can access from the explorer tab in Vitis:

- If you did not create a custom AXI4 peripheral, then the paths are as follows:

- hw/drivers/axi_gpio_asoc_v1_0/src/Makefile

- ps7_cortexa9_0/standalone_ ps7_cortexa9_0/bsp/ ps7_cortexa9_0/libsrc/ axi_gpio_asoc_v1_0/src/Makefile

- zynq_fsbl/zynq_fsbl_bsp/ps7_cortexa9_0/libsrc/axi_gpio_asoc_v1_0/src/Makefile

- If you created a custom AXI4 peripheral, then the paths are (assuming your IP name is AUP_advanced_SoC_v1_0):

- hw/drivers/<AUP_advanced_SoC_v1_0>/src/Makefile

- ps7_cortexa9_0/standalone_ ps7_cortexa9_0/bsp/ ps7_cortexa9_0/libsrc/ < AUP_advanced_SoC_v1_0>/src/Makefile

- zynq_fsbl/zynq_fsbl_bsp/ps7_cortexa9_0/libsrc/<AUP_advanced_SoC_v1_0>/src/Makefile

Make the following changes (underlined in red below) so that the Makefiles are as shown below:

Note: If you are copying and pasting the whole code snippet below into your Makefiles, please ensure that the tab indentations are correct as this may give a Makefile error.

COMPILER=

ARCHIVER=

CP=cp

COMPILER_FLAGS=

EXTRA_COMPILER_FLAGS=

LIB=libxil.a

RELEASEDIR=../../../lib

INCLUDEDIR=../../../include

INCLUDES=-I./. -I${INCLUDEDIR}

INCLUDEFILES=*.h

LIBSOURCES= $(wildcard *.c)

OUTS = *.o

OBJECTS = $(addsuffix .o, $(basename $(wildcard *.c)))

ASSEMBLY_OBJECTS = $(addsuffix .o, $(basename $(wildcard *.S)))

libs:

echo "Compiling axi_gpio_asoc..."

$(COMPILER) $(COMPILER_FLAGS) $(EXTRA_COMPILER_FLAGS) $(INCLUDES) $(LIBSOURCES)

$(ARCHIVER) -r ${RELEASEDIR}/${LIB} ${OBJECTS} ${ASSEMBLY_OBJECTS}

make clean

include:

${CP} $(INCLUDEFILES) $(INCLUDEDIR)

clean:

rm -rf ${OBJECTS} ${ASSEMBLY_OBJECTS}

Save all the modified changes in the Makefile.

# Tasks for this lab

- Hardware modification

  - Add buttons to the hardware design (15 points)

  - Add the Verilog code for the buttons (15 points)

  - Update the constraint file for the buttons (15 points)

- Software modification

  - Update the software code to read the value of button press and the switches (20 points)

- Describe in detail your learning outcomes and challenges of this lab (15 points)

- Video explanation (20 points)